Department of Jobs, Precincts and Regions

Woodvale dust monitoring report q2 2022

31 August 2022

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Jobs, Precincts and Regions

Project Title:	Woodvale Dust Monitoring Report Q2 2022

Title:		Woodvale Dust Monitoring Report Q2 2022

Project: 	MV214940

Document Status:	Version 6

Date of Issue:		 19/08/2022

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 16/08/22 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 17/08/22 |
| Client: | Department of Jobs, Precincts and Regions | Earth Resources Regulation | ERR | 19/08/22 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Jobs, Precincts and Regions for the purpose of monitoring dust at the former Bendigo Mine Woodvale site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1.	Introduction	7

2.	Background	7

2.1	Scope and Objective	7

2.2	Regulatory Environment	9

2.3	What is Dust	9

3.	Measurement of Dust	10

3.1	Site Locations	10

3.2	Deposited Matter	11

3.3	Directional Deposited Dust Monitoring	12

3.4	Metals/Metalloids Monitoring in Dust	13

3.5	Ash Content	13

3.6	Dust Monitoring Standards	13

3.7	Dust Monitoring Program	14

4.	Dust Deposition Results	15

4.1	Woodvale Evaporation Ponds - Q2 2022	15

4.1.1 Deposition of Total Insoluble Matter	15

4.1.2 Mineral Content in Dust	16

4.1.3 Metals/Metalloids in Dust	17

4.1.4 Woodvale Directional Dust Deposition	18

4.2	Review of Quality Assurance and Quality Control	21

5.	Summary of Results	22

6.	Further Considerations	22

7.	References	23

8.	Tables	24

9.	Supplementary Data	33

9.1	Arsenic	33

9.2	Barium	35

9.3	Manganese	37

9.4	Trend Analysis Graphs	39

10.	Appendix A – Supplementary Information	47

11.	Appendix B – Laboratory Reports	50

Table of Figures

Figure 1 - Monitoring Locations at Woodvale Ponds	10

Figure 2 - Control Site Location WVDD07 (BG)	10

Figure 3 - Dust Deposition Gauge WVDD01	11

Figure 4 - Directional Dust Deposition Gauge WVDG06	12

Figure 5 - Woodvale - Total Insoluble Matter (TIM) - Q2 2022	15

Figure 6 - Woodvale - Total Ash Content - Q2 2022	16

Figure 7 - Woodvale - Ash Content/TIM Ratio - Q2 2022	16

Figure 8 - Woodvale - Total Arsenic/Total Solids - Q2 2022	17

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids - Q2 2022 (North/South)	18

Figure 10 - Woodvale - Directional Dust Results - Arsenic Concentrations - Q2 2022 (North/South)	19

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids - Q2 2022 (East/West)	19

Figure 12 - Woodvale - Directional Dust Results - Arsenic Concentrations - Q2 2022 (East/West)	20

Figure 13 - Woodvale - Arsenic in Soluble Fraction - Q2 2022	33

Figure 14 - Woodvale - Arsenic in Insoluble Fraction - Q2 2022	33

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)	34

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)	34

Figure 17 - Woodvale - Barium in Soluble Fraction - Q2 2022	35

Figure 18 - Woodvale - Barium in Insoluble Fraction - Q2 2022	35

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)	36

Figure 20 - Woodvale - Barium in Insoluble Fraction Year Review (2021-22)	36

Figure 21 - Woodvale - Manganese in Soluble Fraction - Q2 2022	37

Figure 22 - Woodvale - Manganese in Insoluble Fraction - Q2 2022	37

Figure 23 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)	38

Figure 24 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)	38

Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022	39

Figure 26 - Woodvale - Total Ash Content - 2021-2022	40

Figure 27 - Woodvale - Ash Content/TIM Ratio - 2021-2022	41

Figure 28 - Woodvale - Total Arsenic/Total Solids - 2021-2022	42

Figure 29 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022	43

Figure 30 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022	44

Figure 31 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022	45

Figure 32 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022	46

Table of Tables

Table 1 - Woodvale Dust Deposition Analysis Results	24

Table 2 - Woodvale Directional Dust Deposition Analysis Results	27

# Executive Summary

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds from the start of January 2021. This report provides results and analysis of data collected from sampling in Q2 April - June 2022.

Earth Resources Regulation (ERR) inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust.

This report analyses the data gathered in Q2 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. During Q3 and Q4, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mine workings at Kangaroo Flat. The site is currently in care and maintenance. ERR is continuing to visually monitor the site by daily site visits (on business days), keeping an eye on weather conditions, and proactively applying a range of dust control measures. The evaporation ponds are the main source of dust at this site, so the ponds are kept in wet condition. Other dust suppression measures include ensuring vegetation cover, mulch application and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, dust monitoring was undertaken at the same locations and a similar methodology and analysis process was followed in this report as per other quarters. Upon routine monthly monitoring, ALS field staff observed all sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring works at Woodvale was to operate and maintain six depositional dust gauges and two directional dust deposition gauges. ERR has since asked ALS to add a further three monitoring sites to the dust monitoring program at Woodvale, i.e. one control site to obtain background data, and two directional dust deposition gauges at the east and west of the Woodvale site, to provide more complete data from directional dust monitoring. These newly added sites have been in operation since 1 October 2021.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data and report exceedances when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

The results for Q2 2022 were comparable to previous results (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021 and ALS Reports for Q3 2021, Q4 2021 and Q1 2022) for majority of the data collected.

As done previously in historic reports, the following questions are answered for Q2 2022:

- Did the dust deposition rates recorded in Q2 2022, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q2 2022, there were four instances where the dust levels were over the 4 g/m2/month criteria used in this report as follows:

- WVDD07 (BG), during April 2022 (6.9 g/m2/month)

- WVDD06 (W), during May 2022 (7.9 g/m2/month)

- WVDD01 (S), during June 2022 (7.6 g/m2/month)

- WVDD06 (W), during June 2022 (8.2 g/m2/month)

However, it should be noted that one of these occurrences was at the background site WVDD07 (BG) and therefore cannot be attributed to the Woodvale Evaporation Ponds. When the data from all source monitoring sites excluding WVDD07 (BG) were averaged for Q2 2022, an average of 2.5 g/m2/month was recorded. This value is below the 4 g/m2/month criteria, and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

The dust levels recorded at WVDD01 (S) and WVDD06 (W) are most likely from surrounding agricultural activities. This observation is reinforced by the background site also recording a higher dust level. In addition, the ponds on the west side of the Woodvale site (where WVDD01 (S) and WVDD06 (W) are located) are maintained in a wet condition by pumping water into these ponds and are less likely to cause dust. The predominant wind during Q2 2022 was from the north, south and west, with least winds from the east. The surrounding land use to the north, south and west is predominantly agricultural farmland with reserved parklands to the east of the Woodvale site. ERR conduct daily site inspections to ensure dust control measures (water cover, dampness, foliage) are adequate.

- Are dust deposition rates recorded in Q2 2022, comparable to available historic results in this area?

Overall, results for Q2 2022 were comparable or a little higher than previous results recorded in the Kralcopic Report for Q4 2018, CDM Smith Report for the first half of 2021, and ALS Reports for Q3 2021, Q4 2021 and Q1 2022.

- Are arsenic concentrations recorded in deposited dust in Q2 2022, comparable to available historic results in this area?

The arsenic concentrations recorded in Q2 2022 were well below the Contaminated Sites Guideline assessment level of 100 mg/kg with all results being below 14 mg/kg. Note that the assessment level of 100 mg/kg is criteria for exposure to contaminated soil, which is used here for context only.

Arsenic, barium and manganese were within the historical ranges (with some results, a little over the mid-range), and the results in Q2 2022 are comparable to results reported in the first half of 2021 in the CDM Smith Report and ALS Reports for Q3 2021, Q4 2021and Q1 2022.

It is noted that in addition to the results above, the directional dust deposition gauge results for Q2 2022 showed that, there is no clear overall net gain in dust or arsenic concentrations resulting from the Woodvale Evaporation Ponds site, with results fluctuating from month to month showing gains and losses.

However, a single elevated net gain (106.7 mg/kg) for total arsenic concentration was recorded in May-22 when comparing the results of the north facing quadrants of the north and south directional gauges. This is due to very low Total Solids being recorded (on this occasion 0.1 g/m2/month) which means arsenic concentration figures can be accentuated when calculations are made with the Total Solids figure in the denominator. No such high calculated concentration figures were seen in any of the other two directional gauges or six depositional dust gauges. An onsite weather monitoring station (which will be installed in the coming months) will help investigate future such elevated results.

# Introduction

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds from the start of January 2021. This report provides results and analysis of data collected from sampling in Q2 2022. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected.  Samples were collected from locations specified by DJPR on a routine monthly basis. These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust. This report analyses the data gathered in Q2 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q1 2022

- ALS Report for Q4 2021

- ALS Report for Q3 2021

- CDM Smith Report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith Report.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mine workings at Kangaroo Flat. The site is currently in care and maintenance. ERR is continuing to visually monitor the site by daily site visits (on business days), keeping an eye on weather conditions, and proactively applying a range of dust control measures. The evaporation ponds are the main source of dust at this site, so the ponds are kept in wet condition. Other dust suppression measures include ensuring vegetation cover, mulch application and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of site rehabilitation.

## Scope and Objective

The scope of dust monitoring works at Woodvale is to operate and maintain seven depositional dust gauges and four directional dust deposition gauges including gathering samples and sending off to the laboratory for analysis, analysing the data from the laboratory and producing quarterly dust monitoring reports consolidating results and comparing to historic and guideline levels.

All depositional dust gauges except for the background gauge (WVDD07 (BG)) are located at their historic locations with the following naming conventions since the dates specified below:

| - WVDD01 (S) Jan 2021 - WVDD02 (N) Jan 2021 - WVDD03 (N) Jan 2021 - WVDD04 (E) Jan 2021 | - WVDD05 (S) Jan 2021 - WVDD06 (W) Jan 2021 - WVDD07 (BG) Sep 2021 |  |
| --- | --- | --- |

The location of the six depositional dust gauges at the Woodvale site can be seen in Figure 1. The location of the background depositional dust gauge is in Figure 2 which serves as a control site.

Similarly, four directional dust deposition gauges were installed since the dates specified below at the following locations:

- WVDG03 (N) Jan 2021

- WVDG04 (E) Oct 2021

- WVDG05 (S) Jan 2021

- WVDG06 (W) Oct 2021

These directional gauges are located next to the depositional dust gauges: WVDD03 (N), WVDD04 (E), WVDD05 (S) and WVDD006 (W).

Laboratory analysis requirements were provided by DJPR and were in line with historical requirements during the previous monitoring program to maintain continuity and comparability of results.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will potentially inform decisions around some aspects of site rehabilitation and any measures required to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q2 2022 data:

- Did the dust deposition rates recorded in Q2 2022, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q2 2022, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q2 2022, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for the air quality in Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from pollution and waste by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in this guidance used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## What is Dust

Dust is typically not classified due to its composition but based on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## Site Locations

Locations of depositional dust gauges are as presented in Figure 1 and Figure 2. Directional depositional dust gauges WVDG03 (N), WVDD04 (E), WVDG05 (S) and WVDD06 (W) have been installed next to the respective depositional gauges as mentioned above.

Figure 1 - Monitoring Locations at Woodvale Ponds

Figure 2 - Control Site Location WVDD07 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air - Determination of particulate matter - Deposited matter - Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel (see sampling bottle in Figure 3 below) and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10:2016)

Figure 3 - Dust Deposition Gauge WVDD01

## Directional Deposited Dust Monitoring

The Standard (AS/NZS 3580.10.2:2013) states that over a given period, particles which impinge on the vertical, internal surface of four collecting cylinders of the directional dust gauge are sampled in collecting pots at the bases of the cylinders. Refer to Figure 4.

The samples are passed through a sieve to remove any extraneous matter (e.g. leaves, insects), with the sieved sample containing the impinged matter subsequently transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of dried insoluble solids determined gravimetrically. Soluble solids are determined by evaporation of the filtrate and gravimetric analysis.

The ash and combustible matter content are determined by loss on combustion of the insoluble solids. Total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass flux rate of impinged matter for each collecting cylinder is calculated from the mass of solids obtained, the slot cross-sectional area and the exposure period.

A directional dust gauge consists of four vertical collecting cylinders, each with a slot of dimensions 45 mm by 340 mm, commencing approximately 10 mm from the top of the cylinder. The edges of each slot shall be at right angles to the width of the slot and not radial (see Figure 4). The cylinders shall be mounted at 90o to each other with the slots facing outwards. An easily detachable collecting pot shall be fitted to the bottom of each collecting cylinder. The combined length of a collecting cylinder and its attached collecting pot shall be at least 640 mm, with the collecting pot having a capacity of approx. 1 L.

The collecting cylinders and pots shall be constructed of chemically resistant material such as polyvinyl chloride. The external diameter of the collecting cylinders shall be 75 mm. The collecting cylinders shall be located with their axes at the corners of a 150 mm square mounting plate. (AS/NZS 3580.10.2:2013)

Figure 4 - Directional Dust Deposition Gauge WVDG06

## Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identify possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to on site contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191  (2007) as shown below which has been adopted in EPA Publication 1961 (2022):

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

# Dust Deposition Results

## Woodvale Evaporation Ponds - Q2 2022

Dust deposition analysis results are presented in Table 1 - Woodvale Dust Deposition Analysis Results and the accompanying Laboratory Certificates of Analysis are presented in Appendix B - Laboratory Reports.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the recommended criteria of less than 4 g/m2/month on 17 of 21 samples during Q2 2022. See Figure 5 below.

Dust deposition rate exceeded the assessment criteria at the sites below:

- WVDD07 (BG), during April 2022 (6.9 g/m2/month)

- WVDD06 (W), during May 2022 (7.9 g/m2/month)

- WVDD01 (S), during June 2022 (7.6 g/m2/month)

- WVDD06 (W), during June 2022 (8.2 g/m2/month)

Dust deposition rates met the background plus 2 g/m2/month criteria on 15 of 18 samples, with the exceptions being the two high readings mentioned above at WVDD06 and WVDD01 in May and June 2022.

When the data from all source monitoring sites except WVDD07 (BG) were averaged for Q2 2022, an average of 2.5 g/m2/month was recorded. This value is well below the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

Figure 5 - Woodvale - Total Insoluble Matter (TIM) - Q2 2022

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. As expected, the increased levels in TIM (Figure 5) generally corresponds with total ash content in Figure 6. Some of the peaks recorded in Figure 5 for May and June, also generally appear in Figure 6.

Figure 6 - Woodvale - Total Ash Content - Q2 2022

To gain a general understanding of what proportion of the sample is ash content i.e. mineral dust, the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Woodvale - Ash Content/TIM Ratio - Q2 2022

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, barium and manganese concentrations for Q2 2022 were within historical ranges and results being comparable to results reported in the CDM Smith Report for the first half of 2021 and in the ALS Reports for Q3 2021, Q4 2021 and Q1 2022.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below Figure 8 in mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month dividing by the total solids g/m2/month.

This ratio has been plotted as this provides a value which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series - Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 8 - Woodvale - Total Arsenic/Total Solids - Q2 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q2 ranged from 1.0 to 13.8 mg/kg with a mean of 6.3 mg/kg across all depositional monitoring sites (excluding the background monitoring site). The mean is well below the 100 mg/kg assessment level mentioned above.

Highest concentrations were recorded in Apr-22 at WVDD05 (S) (13.7 mg/kg) and WVDD06 (W) (13.6 mg/kg) and in Jun-22 at WVDD03 (N) (11.6 mg/kg) and WVDD05 (S) (13.8 mg/kg). Again, these values are still well below the 100 mg/kg assessment level mentioned above.

A number of samples (10 out of 18 samples) at the site recorded marginally higher arsenic concentration levels than the samples taken from the background control site. This can be expected because of the known contamination at the Woodvale site, but these levels are only marginally higher because maximum possible dust control measures have been applied at the Woodvale site.

## 4.1.4 Woodvale Directional Dust Deposition

Results for directional dust deposition gauges are summarised in Table 2 - Woodvale Directional Dust Deposition Analysis Results, with Laboratory Analytical Results attached in Appendix B. As mentioned previously, for the purpose of continuity, the same approach for review of dust from the directional dust deposition gauges as in the CDM Smith Report has been applied here for Q2 2022 results.

The results from the northerly and southerly facing quadrants at the two directional dust deposition gauges WVDG03 (N) and WVDG05 (S) have been compared in Figure 9. Figure 9 shows the net gain or loss in dust deposition downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

From the graph below (Figure 9) it can be seen that there is a small net gain in dust observed through the Q2 2022 monitoring period, with results varying from -0.1 g/month to 0.3 g/month.

Such small gains can be expected and can be influenced by both the Woodvale site and environmental conditions. Note that there is agricultural farmland to the north, south and west of the Woodvale site and reserved parklands to the east of the Woodvale site, which can all contribute to the dust levels at these sampling locations. Furthermore, the predominant winds during this quarter were from the north and the south which would have also influenced these results. The net gains were very small implying that the dust control measures at this site may be working well.

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids - Q2 2022 (North/South)

Similarly, the concentration of arsenic was compared using the same methodology as above, and it was found that low net gains and losses occurred for 5 of the 6 values recorded during Q2 2022 with these levels being comparable to what was observed in 2021 and Q1 2022.

Figure 10 presents the net gain and loss in total arsenic concentration downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

Figure 10 - Woodvale - Directional Dust Results - Arsenic Concentrations - Q2 2022 (North/South)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 10 recorded an elevated net gain (106.7 mg/kg) for total arsenic concentration in May-22 from the results of the north facing quadrants of the north and south directional gauges. This is due to very low Total Solids being recorded (on this occasion 0.1 g/m2/month) which means arsenic concentration figures can be accentuated when calculations are made with the Total Solids figure in the denominator. On this occasion the calculation was 10.67 µg/m2/month divided by 0.1 g/m2/month which gave 106.7 mg/kg. No such high concentration figures were seen in any of the other two directional gauges or six depositional dust gauges. Although, this elevated net gain result may suggest that the site may be a source of increased arsenic concentration in dust, it should be noted here that this high concentration figure has occurred because the Total Solids at this location was especially low, causing the calculated arsenic concentration result to be accentuated. An onsite weather monitoring station (which will be installed in the coming months) will help investigate future such elevated results.

Figure 11 below displays the net gain or loss both upwind and downwind from east and west and no clear pattern is identified during Q2 2022 with both gains and losses occurring throughout the period.

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids - Q2 2022 (East/West)

Error! Reference source not found. presents the net gains and losses in total arsenic concentration downwind of the site compared to upwind of the site, for both east and west facing quadrants in the two directional dust deposition gauges being considered here. Similarly, the concentration of arsenic was compared using the same methodology as for the north/south gauges, and it was found that net losses occurred in 5 of the 6 readings observed during Q2 2022.

Figure 12 - Woodvale - Directional Dust Results - Arsenic Concentrations - Q2 2022 (East/West)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Review of Quality Assurance and Quality Control

Field sampling was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in 2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections. In summary, the overall results for Q2 2022 were comparable to previous results.

The key findings for Q2 2022 are as follows:

- There were four instances where the dust level exceeded the 4 g/m2/month criteria used in this report.  One of these instances occurred at the Background site (WVDD07 (BG)) and is not attributed to the Woodvale Evaporation Ponds.

- WVDD07 (BG), during April 2022 (6.9 g/m2/month)

- WVDD06 (W), during May 2022 (7.9 g/m2/month)

- WVDD01 (S), during June 2022 (7.6 g/m2/month)

- WVDD06 (W), during June 2022 (8.2 g/m2/month)

- Arsenic, barium and manganese were within the historical ranges, with results being comparable to results reported in previous 2021 and Q1 2022 reports.

- The control site results from Q2 2022 confirm the previous observation that the arsenic concentration levels recorded in the dust samples from the site are generally slightly higher than the background arsenic levels in the area but is still much lower than the Contaminated Sites Guideline criteria.

- The results from the directional dust monitoring gauges showed no clear patterns although an elevated level was recorded of net gain of arsenic concentration (106.7 mg/kg) which may have been caused by windy conditions of the autumn months, predominant northerly winds and relatively lower rainfall.

# Further Considerations

ERR have advised that they will continue to monitor the site with daily visits (on business days) and ensure that a range of dust control measures are applied during the current care and maintenance phase of this rehabilitation project. ERR is also reviewing the effectiveness of dust control measures and monitoring on an ongoing basis and adopting a continuous improvement approach to dust minimisation and monitoring.

The installation of an onsite weather monitoring station will help investigate any future elevated results.

# References

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q4 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q1 2022 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Woodvale Evaporation Ponds - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA Vic Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA NSW Publication (2017), Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, January 2017.

- EPA Vic Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Vic Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report - Woodvale Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Peter J Ramsay and Associates (2018), Woodvale Evaporation Ponds Complex, Environmental Audit of the Risk of any Possible Harm or Detriment to the Environment Pursuant to Part IXD (Section 53V) of the Environment Protection Act 1970, 29 June 2018. (Environmental Audit Report)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

# Tables

Table 1 - Woodvale Dust Deposition Analysis Results

Table 2 - Woodvale Directional Dust Deposition Analysis Results

# Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## Arsenic

Figure 13 - Woodvale - Arsenic in Soluble Fraction - Q2 2022

Figure 14 - Woodvale - Arsenic in Insoluble Fraction - Q2 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Barium

Figure 17 - Woodvale - Barium in Soluble Fraction - Q2 2022

Figure 18 - Woodvale - Barium in Insoluble Fraction - Q2 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)

Figure 20 - Woodvale - Barium in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Manganese

Figure 21 - Woodvale - Manganese in Soluble Fraction - Q2 2022

Figure 22 - Woodvale - Manganese in Insoluble Fraction - Q2 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 23 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)

Figure 24 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Trend Analysis Graphs

Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022

Figure 25 shows TIM for all months in 2022 and the first two quarters of 2022. The review of Q2 2022 data shows similar data trends across all sites with only a few exceptions which have been highlighted in previous quarterly reports. In Q2 2022, elevated levels were recorded at WVDD07 (BG) for Apr-22, WVDD06 for May-22 and Jun-22 and WVDD01 for Jun-22. This may have been influenced by environmental conditions including but not limited to strong wind conditions and dryer ground conditions from low rainfall recorded. The overall average from all monitoring locations is also slightly higher being 2.5 g/m2/month for Q2 2022 (excluding the background control site), compared to 1.7 g/m2/month for all quarters in 2021 and Q1 2022 (excluding the background control site) but is comparable to previous results. In comparison, it can be seen in Figure 25 during Q2 2021 there were 3 exceedances that occurred at sites WVDD01, WVDD03 and WVDD06, all recording results over the 4g/m2/month level. During the Q2 2022 field visits, ALS observed no variation in site conditions whilst undertaking monthly site visits which could have contributed to higher levels recorded.

Figure 26 - Woodvale - Total Ash Content - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 26 shows the Total Ash Content for all months in 2021and the first two quarters of 2022 with Q2 monthly figures being at similar levels to the levels observed in Q2 2021.

Figure 27 - Woodvale - Ash Content/TIM Ratio - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 27 shows all results collected and sampled for 2021 as well as results for Q1 and Q2 of 2022. It can be seen in the results represented above that the Q2 2022 results are comparable to other quarters monitored.

Figure 28 - Woodvale - Total Arsenic/Total Solids - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 28 above presents the ratio of Total Arsenic/Total Solids which shows that the Q2 2022 results are comparable to previous results in 2021 and Q1 2022.

Figure 29 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022

Figure 29 above displays the results from the previous 18 months, no definitive pattern is apparent throughout this period with small net gains and losses from both the upwind and downwind quadrants being observed. The southerly facing quadrant does have greater gains in 13 of 18 months and the northerly facing quadrant having gains in 9 of 18 months. However, the overall gains from the site are showing to be very low levels.

Figure 30 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 30 shows the net gains and losses in total arsenic concentration for the north and south quadrants from the previous 18 months. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed. The elevated net gain level discussed in Section 4.1.4 of this report appears at May-22 in this graph.

Figure 31 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022

Figure 31 above displays the net gain or loss both upwind and downwind from east and west for total solids, and no clear pattern is identified during the last 9 months with both gains and losses occurring throughout the period.

Figure 32 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022

Figure 322 shows the net gains and losses in total arsenic concentration for the east/west quadrants from the previous 9 months. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports