Department of Jobs, Precincts and Regions

Woodvale dust monitoring report q4 2021

03 MARCH 2022

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Jobs, Precincts and Regions

Project Title:	Woodvale Dust Monitoring Report Q4 2021

Title:		Woodvale Dust Monitoring Report Q4 2021

Project: 	MV214940

Document Status:	Final

Date of Issue:		  03/03/2022

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 08/02/22 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 08/02/22 |
| Client: | Department of Jobs, Precincts and Regions | Earth Resources Regulation | ERR | 03/03/22 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Jobs, Precincts and Regions for the purpose of monitoring dust at the former Bendigo Mine Woodvale site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1.	Introduction	6

2.	Background	6

2.1	Scope and Objective	6

2.2	Regulatory Environment	8

2.3	What is Dust	8

3.	Measurement of Dust	9

3.1	Site Locations	9

3.2	Deposited Matter	10

3.3	Directional Deposited Dust Monitoring	11

3.4	Metals/Metalloids Monitoring in Dust	12

3.5	Ash Content	12

3.6	Dust Monitoring Standards	12

3.7	Dust Monitoring Program	13

4.	Dust Deposition Results	13

4.1	Woodvale Evaporation Ponds – Q4 2021	13

4.1.1 Deposition of Total Insoluble Matter	13

4.1.2 Mineral Content in Dust	15

4.1.3 Metals/Metalloids in Dust	17

4.1.4 Woodvale Directional Dust Deposition	18

4.2	Review of Quality Assurance and Quality Control	22

5.	Summary of Results	23

6.	Further Considerations	23

7.	References	24

8.	Tables	25

9.	Supplementary Data	37

9.1	Arsenic	37

9.2	Barium	39

9.3	Manganese	41

10.	Appendix A – Supplementary Information	43

11.	Appendix B – Laboratory Reports	46

Table of Figures

Figure 1 - Monitoring Locations at Woodvale Ponds	9

Figure 2 - Control Site Location WVDD07BG	9

Figure 3 - Dust Deposition Gauge WVDD01	10

Figure 4 - Directional Dust Deposition Gauge WVDG06	11

Figure 5 - Woodvale - Total Insoluble Matter (TIM) – Q4 2021	14

Figure 6 - Woodvale - Total Insoluble Matter (TIM) - 2021	14

Figure 7 - Woodvale - Total Ash Content	15

Figure 8 - Woodvale - Total Ash Content (2021)	15

Figure 9 - Woodvale - Ash Content/TIM Ratio	16

Figure 10 - Woodvale - Ash Content/TIM Ratio (2021)	16

Figure 11 - Woodvale - Total Arsenic/Total Solids - Q4 2021	17

Figure 12 - Woodvale - Total Arsenic/Total Solids - 2021 Data	18

Figure 13 - Woodvale - Directional Dust Deposition Results - Total Solids	19

Figure 14 - Woodvale - DDG Total Solids Comparison North/South	19

Figure 15 - Woodvale - Directional Dust Deposition Results - Arsenic Concentrations	20

Figure 16 - Woodvale - Arsenic - North/South (2021)	20

Figure 17 - Woodvale - DDG Total Solids Comparison - East/West	21

Figure 18 - Woodvale - Arsenic - East/West	21

Figure 19 - Woodvale - Arsenic in Soluble Fraction	37

Figure 20 - Woodvale - Arsenic in Insoluble Fraction	37

Figure 21 - Woodvale - Arsenic in Soluble Fraction Year Review	38

Figure 22 - Woodvale - Arsenic in Insoluble Fraction Year Review	38

Figure 23 - Woodvale - Barium in Soluble Fraction	39

Figure 24 - Woodvale - Barium in Insoluble Fraction	39

Figure 25 - Woodvale - Barium in Soluble Fraction Year Review	40

Figure 26 - Woodvale - Barium in Insoluble Fraction Year Review	40

Figure 27 - Woodvale - Manganese in Soluble Fraction	41

Figure 28 - Woodvale - Manganese in Insoluble Fraction	41

Figure 29 - Woodvale - Manganese in Soluble Fraction Year Review	42

Figure 30 - Woodvale - Manganese in Insoluble Fraction Year Review	42

Table of Tables

Table 1 - Dust Deposition - Woodvale Evaporation Ponds	25

Table 2 - Woodvale Directional Dust Deposition	29

# Executive Summary

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds from the start of January 2021. This report provides results and analysis of data collected from sampling in Q4 2021.

Earth Resources Regulation (ERR) inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust.

This report analyses the data gathered in Q4 2021 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. During Q3 and Q4, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory – NATA accredited) and drafted the dust monitoring reports.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold water extracted from historic underground mining operations in Kangaroo Flat. The site is currently in care and maintenance. As noted in Q3 the care and maintenance for Q4 continued with a consistent approach to reduce dust levels at the Woodvale mine location. ERR is also applying a range of dust control measures at the Bendigo Mine sites, including mulching exposed areas, ensuring there is vegetation cover, watering and applying dust suppression polymer as needed.

To maintain continuity and comparability of data, dust monitoring was undertaken at the same locations and a similar methodology and analysis process was followed in this report as per other quarters. Upon routine monthly monitoring ALS field staff observed all sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring works at Woodvale was to operate and maintain six depositional dust gauges and two directional dust deposition gauges. ERR has since asked ALS to add a further three monitoring sites to the dust monitoring program at Woodvale, i.e. one control site to obtain background data, and two directional dust deposition gauges at the east and west of the Woodvale site, to provide more complete data from directional dust monitoring. These newly added sites have been in operation since 1 October 2021.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

The results for Q4 2021 were comparable to previous results (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021 and ALS Report for Q3 2021) for majority of the data collected. Results trended less than what was observed in the first half of 2021.

As done previously in historic reports, the following questions are answered for Q4 2021:

- Did the dust deposition rates recorded in Q4 2021, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q4 2021, there was no instances where the dust levels were over the 4 g/m2/month criteria used in this report.

- Are dust deposition rates recorded in Q4 2021, comparable to available historic results in this area?

Overall results for Q4 2021 were comparable to previous results (Kralcopic Report in 2018 and CDM Smith Report in 2021), or less than what was observed in the first half of 2021.

- Are arsenic concentrations recorded in deposited dust in Q4 2021, comparable to available historic results in this area?

Arsenic, barium and manganese were within the historical ranges, and the results in Q4 2021 are comparable to or lower than results reported in the first half of 2021 in the CDM Smith Report 2021 and ALS Q3 2021 Report.

The notable high result of arsenic in November 2021 at WVDD03 (N) appears to be an anomaly because no logical explanation was found for this unusually high figure. Further investigation into the results at WVDD03 (N) location for January 2022 (which will be reported on in the Q1 2022 Dust Monitoring Report) showed the same parameter was 13.1 mg/kg, which is more in alignment with other data for this location. This confirms that the value of 162.1 mg/kg is genuinely erroneous.

The control site results from Q4 2021 confirm the previous observation that the arsenic concentration levels recorded in the dust samples at the Woodvale Ponds are generally representative of the background arsenic levels in the area.

In addition to the results above, the directional dust deposition gauge results for Q4 2021 showed that, there is no clear net increase in dust or arsenic concentrations resulting from the Woodvale Evaporation Ponds, with results fluctuating from month to month.

# Introduction

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds from the start of January 2021. This report provides results and analysis of data collected from sampling in Q4 2021. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected.  Samples were collected from locations specified by DJPR on a routine monthly basis. These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust. This report analyses the data gathered in Q4 2021 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q3 2021

- CDM Smith Report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith report.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold water extracted from historic underground mining operations in Kangaroo Flat. The site is currently in care and maintenance. To keep dust levels down, some of the ponds are being filled with water. ERR is also applying a range of dust control measures at the Bendigo Mine sites, including mulching exposed areas, ensuring there is vegetation cover, watering and applying dust suppression polymer as needed.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of site rehabilitation.

## Scope and Objective

The scope of dust monitoring works at Woodvale is to operate and maintain seven depositional dust gauges and four directional dust deposition gauges.

All depositional dust gauges except for the background gauge (WVDD07BG) are located at their historic locations with the following naming conventions since the dates specified below:

| - WVDD01 (S) Jan 2021 - WVDD02 (N) Jan 2021 - WVDD03 (N) Jan 2021 - WVDD04 (E) Jan 2021 | - WVDD05 (S) Jan 2021 - WVDD06 (W) Jan 2021 - WVDD07BG Sep 2021 |
| --- | --- |

Similarly, directional dust deposition gauges were installed since the dates specified below at the following locations:

- WVDG03 (N) Jan 2021

- WVDG04 (E) Oct 2021

- WVDG05 (S) Jan 2021

- WVDG06 (W) Oct 2021

These directional gauges are located next to the existing gauges WVDD03 (N), WVDD04 (E), WVDD05 (S) and WVDD006 (W).

Laboratory analysis requirements were provided by DJPR and were in line with historical requirements during the previous monitoring program to maintain continuity and comparability of results.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will potentially inform decisions around some aspects of site rehabilitation and any measures required to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q4 2021 data:

- Did the dust deposition rates recorded in Q4 2021, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q4 2021, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q4 2021, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for the air quality in Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from pollution and waste by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM).

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment. So the focus is on applying effective control measures, and monitoring is to determine the effectiveness of control measures.

## What is Dust

Dust is typically not classified due to its composition but based on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## Site Locations

Locations of depositional dust gauges are per Figure 1. Directional depositional dust gauges WVDG03 (N), WVDD04 (E), WVDG05 (S) and WVDD06 (W) have been installed next to the respective depositional gauges.

Figure 1 - Monitoring Locations at Woodvale Ponds

Figure 2 - Control Site Location WVDD07BG

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air – Determination of particulate matter – Deposited matter – Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10:2016)

Figure 3 - Dust Deposition Gauge WVDD01

## Directional Deposited Dust Monitoring

The Standard (AS/NZS 3580.10.2:2013) states that over a given period, particles which impinge on the vertical, internal surface of four collecting cylinders of the directional dust gauge are sampled in collecting pots at the bases of the cylinders. Refer to Figure 4.

The samples are passed through a sieve to remove any extraneous matter (e.g. leaves, insects), with the sieved sample containing the impinged matter subsequently transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of dried insoluble solids determined gravimetrically. Soluble solids are determined by evaporation of the filtrate and gravimetric analysis.

The ash and combustible matter content are determined by loss on combustion of the insoluble solids. Total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass flux rate of impinged matter for each collecting cylinder is calculated from the mass of solids obtained, the slot cross-sectional area and the exposure period.

A directional dust gauge consists of four vertical collecting cylinders, each with a slot of dimensions 45 mm by 340 mm, commencing approximately 10 mm from the top of the cylinder. The edges of each slot shall be at right angles to the width of the slot and not radial (see Figure 4). The cylinders shall be mounted at 90° to each other with the slots facing outwards. An easily detachable collecting pot shall be fitted to the bottom of each collecting cylinder. The combined length of a collecting cylinder and its attached collecting pot shall be at least 640 mm with the collecting pot having a capacity of approx. 1 L.

The collecting cylinders and pots shall be constructed of chemically resistant material such as polyvinyl chloride. The external diameter of the collecting cylinders shall be 75 mm. The collecting cylinders shall be located with their axes at the corners of a 150 mm square mounting plate. (AS/NZS 3580.10.2:2013)

Figure 4 - Directional Dust Deposition Gauge WVDG06

## Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identify possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to on site contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191  (2007), as shown below:

The following points apply to the criteria:

- Results of monitoring should not exceed 4g/m2/month (no more than 2g/m2/month above background) as a monthly average.

- The 2g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4g/m2/month criteria is used when no baseline data exists.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

# Dust Deposition Results

## Woodvale Evaporation Ponds – Q4 2021

Dust deposition analysis results are presented in Table 1 - Dust Deposition - Woodvale Evaporation Ponds.  Laboratory Certificates of Analysis are presented in Appendix B – Laboratory Reports.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the recommended criteria of  less than 4 g/m2/month for all 21 samples during Q4 2021. See Figure 5 below.

Dust deposition rate at the background site WVDD07BG was higher than all readings throughout the period except WVDD03 Oct 2021 result. However, WVDD03 Oct 2021 result was still under the background plus 2g/m2/month criteria, therefore meeting still meeting the Mining PEM criteria.

When the data from all monitoring sites were averaged for Q4 2021, an average of 1.3 g/m2/month was recorded. This value is below the inferred background level suggested within the Mining PEM.

Figure 5 - Woodvale - Total Insoluble Matter (TIM) – Q4 2021

Red dotted line is the criteria used for the purpose of this report.

Figure 6 - Woodvale - Total Insoluble Matter (TIM) - 2021

Figure 6 shows TIM for all months in 2021. Comparison of Q4 data with this period show that less exceedances of the recommended criteria level have occurred with no exceedances being observed in Q4 2021, compared to five during the first three quarters of 2021. The overall average from all monitoring locations is also lower being 1.3 g/m2/month for Q4, compared to 1.7 g/m2/month for Q1, Q2 and Q3 2021. ALS observed no variation in site conditions whilst undertaking monthly site visits.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 7. As expected, the increased levels in TIM (Figure 5) generally corresponds with total ash content in Figure 7. This suggests increases in TIM are influenced by elevated levels of mineral dust. The peaks recorded in Figure 5 for October and December also generally appear in Figure 7.

Figure 7 - Woodvale - Total Ash Content

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 8 shows the Total Ash Content for all quarters in 2021 with Q4 monthly figures being on the lower end of the monthly figures for other quarters of 2021.

Figure 8 - Woodvale - Total Ash Content (2021)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

To gain a general understanding of what proportion of the sample is ash content i.e. mineral dust, the ratio of total ash content and TIM has been plotted in Figure 9.

Figure 9 - Woodvale - Ash Content/TIM Ratio

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 10 shows the Ash Content/TIM Ratio for all the quarters in 2021. As with Figure 8 above, this ratio values for Q4 2021 are also showing on the lower end as compared to other quarters in 2021.

Figure 10 - Woodvale - Ash Content/TIM Ratio (2021)

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, barium and manganese concentrations for Q4 2021 were within historical ranges and results being comparable to or lower than results reported in the first half of 2021 in the CDM Smith Report and in ALS Q3 2021 Report. The only exception to this can be seen in Figure 11 was at site WVD003 (N). Field staff observed no changes at the site and no other contributing factors.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic has been displayed below Figure 11 as mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month divided by the total solids g/m2/month.

This ratio has been plotted as this provides a value which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series – Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 11 - Woodvale - Total Arsenic/Total Solids - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q4 ranged from 0.3 to 162.1 mg/kg with a mean of 12.1 mg/kg across all depositional monitoring sites. The mean is well below the 100 mg/kg assessment level mentioned above. The notable high result of arsenic in November 2021 at WVDD03 (N) appears to be an anomaly because no logical explanation was found for this unusually high figure. Further investigation into the results at WVDD03 (N) location for January 2022 (which will be reported on in the Q1 2022 Dust Monitoring Report) showed the same parameter was 13.1 mg/kg, which is more in alignment with other data for this location. This confirms that the value of 162.1 mg/kg is genuinely erroneous.

When comparing arsenic concentrations to those recorded at the background site (WVDD07BG) it can be seen that levels recorded are similar or slightly elevated but not significantly higher (except for WVDD03 Nov 21). As ALS noted no observed changes on site, ALS cannot determine any influences that may have contributed to the higher result.

There is no definitive pattern across the sites indicating that arsenic concentrations are unlikely to be influenced by a single source with multiple factors likely to be having an influence. Highest concentrations recorded in this period were WVDD05 (15.0 mg/kg), WVDD03 (162.1 mg/kg) and WVDD06 (19.0 mg/kg).

Figure 12 - Woodvale - Total Arsenic/Total Solids - 2021 Data

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

By comparing the graphs in Figure 11 and Figure 12 above, it can be concluded that these results are generally in line with previous results and in fact the figures recorded for Q4 2021 were overall lower (with the exception of WVDD03 Nov-21) than the figures recorded in the first three quarters of 2021. This could be interpreted as the dust from the Woodvale site may have been lower in Q4 2021 as compared to the first three quarters of 2021.

## 4.1.4 Woodvale Directional Dust Deposition

Results for directional dust deposition gauges are summarised in Table 2 - Woodvale Directional Dust Deposition, with laboratory analytical results attached in Appendix B. As mentioned previously, for the purpose of continuity, the same approach for review of dust from the directional dust deposition gauges as in the CDM Smith report has been applied here for Q4 2021 results.

The results from the northerly and southerly facing quadrants at the two directional dust deposition gauges WVDG03 (N) and WVDG05 (S) have been compared in Figure 13. Figure 13 shows the net gain or loss in dust deposition downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

From the graph below (Figure 13) it can be seen that there is a small net increase in dust observed through the Q4 2021 monitoring period, with results varying but staying under 1.0 g/month. Dust deposition during Q4 for both North/South gauges recorded a positive net gain. Possible causes could be contributed to environmental conditions. The south facing gauge is influenced by forest whereas typical land uses in the north are predominately agricultural.

Figure 13 - Woodvale - Directional Dust Deposition Results - Total Solids

Figure 14 below displays the results from the previous 12 months (2021), no definitive pattern is apparent throughout this period with small net gains and losses from both the upwind and downwind quadrants being observed. The southerly facing quadrant does have greater gains in 9 of 12 months.

Figure 14 - Woodvale - DDG Total Solids Comparison North/South

Similarly, the concentration of arsenic was compared using the same methodology as above, and it was found that low net increase and decreases occurred throughout the period with observed levels being less than what was observed in the first half of 2021. Figure 15 presents the net gain and loss in total arsenic (mg) downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

Figure 15 - Woodvale - Directional Dust Deposition Results - Arsenic Concentrations

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 shows the results from the previous 12 months (2021). No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

Figure 16 - Woodvale - Arsenic - North/South (2021)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Two new directional dust deposition gauges WVDG06 (W) and WVDG04 (E) were installed during September 2021 and data for Q4 is displayed below using the same methodology as that used for the north and south directional depositional gauges. The east and west facing quadrants were compared to assess dust gains or losses from the east and west.

Figure 17 below displays the net gain or loss both upwind and downwind from east and west and no clear pattern is identified during Q4 with both gains and losses occurring throughout the period.

Figure 17 - Woodvale - DDG Total Solids Comparison - East/West

Figure 18 - Woodvale - Arsenic - East/West

Figure 18 presents the net gains and losses in total arsenic (mg) downwind of the site compared to upwind of the site, for both east and west facing quadrants in the two directional dust deposition gauges being considered here. Similarly, the concentration of arsenic was compared using the same methodology as for the north/south gauges, and it was found that low net gains and losses occurred in Figure 18.

Directional deposition dust results indicate that the site does not appear to be a source of increased arsenic levels which is consistent with the control site findings above.

## Review of Quality Assurance and Quality Control

Field sampling was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in mid-2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections. In summary, the overall results for Q4 2021 were comparable to previous results or less than what was observed in the previous 2021 quarters.

The key findings for Q4 2021 are as follows:

- There were no instances where the dust level exceeded the 4 g/m2/month criteria used in this report.

- There were no instances observed where dust levels were greater than 2g/m2/month above the background site (WVDD07BG). One reading (WVDD03 in Oct 2021) was higher than the background site but did not exceeding the background plus 2g/m2/month tolerance.

- Arsenic, barium and manganese were within the historical ranges, and results being comparable to or lower than results reported in previous 2021 reports.

- The control site results from Q4 2021 confirm the previous observation that the arsenic concentration levels recorded in the dust samples are generally representative of the background arsenic levels in the area.

# Further Considerations

In this quarter i.e. Q4 2021, the weather was favourable to keep the dust levels low with high rainfall observed. ERR has also advised that they will continue to monitor the site and ensure that a range of dust control measures are applied during the current care and maintenance phase of the rehabilitation project reviewing the effectiveness of control measures and monitoring on an ongoing basis.

# References

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q3 2021 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Woodvale Evaporation Ponds - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA (Vic) Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA Publication 1961 (2021), Guideline for Assessing and Minimising Air Pollution in Victoria (Draft), May 2021. (Assessing and Minimising Guideline)

- EPA Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report – Woodvale Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Peter J Ramsay and Associates (2018), Woodvale Evaporation Ponds Complex, Environmental Audit of the Risk of any Possible Harm or Detriment to the Environment Pursuant to Part IXD (Section 53V) of the Environment Protection Act 1970, 29 June 2018. (Environmental Audit Report)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

# Tables

Table 1 - Dust Deposition - Woodvale Evaporation Ponds

Table 2 - Woodvale Directional Dust Deposition

# Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions.

## Arsenic

Figure 19 - Woodvale - Arsenic in Soluble Fraction

Figure 20 - Woodvale - Arsenic in Insoluble Fraction

Figure 21 - Woodvale - Arsenic in Soluble Fraction Year Review

Figure 22 - Woodvale - Arsenic in Insoluble Fraction Year Review

## Barium

Figure 23 - Woodvale - Barium in Soluble Fraction

Figure 24 - Woodvale - Barium in Insoluble Fraction

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 25 - Woodvale - Barium in Soluble Fraction Year Review

Figure 26 - Woodvale - Barium in Insoluble Fraction Year Review

## Manganese

Figure 27 - Woodvale - Manganese in Soluble Fraction

Figure 28 - Woodvale - Manganese in Insoluble Fraction

Figure 29 - Woodvale - Manganese in Soluble Fraction Year Review

Figure 30 - Woodvale - Manganese in Insoluble Fraction Year Review

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports